#### PRODUCT COM #569

# June 23, 2010

TO:	Field Sales, Motor Representatives, Application Engineering, Product Managers

FROM:	Rich Steele

## SUBJECT: 	56 Frame Leakage Current Shield (LCS)

I would like to present the Leakage Current Shield (LCS) motor platform. This new 56 Frame product will attenuate leakage current, which causes nuisance GFCI tripping in applications using 3 phase motors with a drive.

In order to reduce leakage currents to an acceptable level, an insulator comprised of metallic layers sandwiched between two insulative layers are added to each stator (Figure 1).

*Available in most 56 Frame constructions and features

# Figure 1                                             TEFC                                       Dripproof

LCS MOTOR PLATFORM

- Platform:  CASA NEMA 56 Frame, 3PH product line

- Voltages:  230V, 460V

- Construction: TEFC, Open, DP, TEAO

- Speeds:  4-Pole, 6-Pole

- Mountings:  Welded base, Resilient base, 56 Frame C-Face, Extended Clamp Bolts

### APPLICATIONS

- Commercial Mixers, Pool Pumps, Restaurant Equipment

- Any application using a 3 phase motor with a VFD on a GFCI

### BENEFITS/ADDERS

- Cost-savings due to reduced service calls (Adding a new GFCI or isolation transformer)

- Adder of $50-$75 depending on volumes and stack size

For further questions or inquiries, please contact Rich Steele.